New Guidance Document AD M Volume1 – Dwellings

A full version of this new Approved Document can be found using the link below;

https://www.gov.uk/government/uploads/system/uploads/attachment_data/file/354091/02__140731__HSR_Supporting_Doc1__Access.pdf

This document shows briefly the elements that affect doors, windows and stairs.

Section 1 Visitable Dwellings

| Requirement | Limits on application |
| --- | --- |
| Category 1 - Visitable dwellings M4(1) Reasonable provision should be made for people to : (a) gain access to; and (b) use The building and its facilities | Requirement M4(1) does not apply to: (a) an extension of or material alteration of a dwelling (b) any part of the building which is used solely to enable the building or any service or fitting in the building to be inspected, repaired or maintained |

Requirements M4(1) will be met when a new dwelling makes reasonable provision for most people, including wheelchair users, to approach and enter the dwelling and to gain access to habitable rooms and sanitary facilities on the entrance storey.

Section 1A: Approach to the dwelling

Communal entrances 	minimum clear width and accessible threshold

Communal stairs - 	Where the dwelling is located on an upper floor and does not have lift access, the stair meets the requirements of Part K (Protection from falling, collision and impact) for a general access stair;

where the dwelling is located on an upper floor and does have lift access, the stair meets the requirements of Park K for a utility stair.

Section 1B: Private entrances and spaces within the dwelling

Private entrances 		minimum clear width and accessible threshold

Door and hall widths 	minimum clear width

Private stairs and	 	where unavoidable, internal steps or stairs comply with the

changes of level within 	provisions of Part K (Protection from falling, impact and the entrance storey 		collision) for a private stair.

Section 2 Accessible and adaptable dwellings

| Optional Requirement | Limits on application |
| --- | --- |
| PART M ACCESS TO AND USE OF BUILDINGS Category 2 - Accessible and adaptable dwellings M4 (2) Optional requirement. (1) Reasonable provision must be made for people to— (a) gain access to, and (b) use, the dwelling and its facilities. (2) The provision made must be sufficient to allow adaptation of the dwelling— (a) to meet changing needs of the occupants over time, or (b) to meet the needs of occupants with differing needs. | Optional requirement M4(2)— (a) may apply only in relation to new dwellings; (b) will apply in substitution for requirement M4(1); (c) does not apply where optional requirement M4(3) applies; (d) does not apply to any part of a building that is used solely to enable the building or any service or fitting in the building to be inspected, repaired or maintained. |

Optional Requirement M4(2) will be met where a new dwelling makes reasonable provision for access for most people and incorporates features that make it potentially suitable for a wide range of occupants, including older people, those with reduced mobility and some wheelchair users.

Section 2A: Approach to the dwelling

Principle communal entrance 	minimum clear width and accessible threshold

300mm “nib” provided to leading edge of door and depth of reveal on leading side of the door not to exceed 200mm

Other communal doors	above apply to every other communal door, or gate, along the approach route.

Communal stairs 	Ensure that the principal stair that give access to the dwelling meets the requirements of Part K for a general access stair.

Section 2B: Private entrances and spaces within the dwelling

Principal private entrance 	minimum clear width and accessible threshold

and alternative entrance 	300mm “nib” provided to leading edge of door and depth of reveal on leading side of the door not to exceed 200mm

Circulation areas and		minimum clear width and accessible threshold

internal doorways			300mm “nib” provided to leading edge of door

Private stairs and 			The stair from the entrance storey to the storey

changes of level within  		above (or below) and any stair within the storey above

the dwelling 	(or below) has a clear width of at least 850mm when measured at a height of 450mm above the pitch line of the treads (ignoring any newel post) and all stairs meet the provisions of Part K for private stairs.

Living, kitchen and 			glazing to the principal window to the principal living eating areas 				area starts no higher than 850mm above floor level

Section 3 Wheelchair user dwellings

| Optional Requirement | Limits on application |
| --- | --- |
| Category 3 -Wheelchair user dwellings M4(3) Optional Requirement (1) Reasonable provision must be made for people to— (a) gain access to, and (b) use, the dwelling and its facilities. (2) The provision made must be sufficient— (a) to meet the needs of occupants who use wheelchairs; or (b) to allow simple adaptation of the dwelling to meet the needs of occupants who use wheelchairs.. | Optional requirement M4(3)— (a) may apply only in relation to a dwelling that is erected; (b) will apply in substitution for requirement M4(1); (c) does not apply where optional requirement M4(2) applies; (d) does not apply to any part of a building that is used solely to enable the building or any service or fitting in the building to be inspected, repaired or maintained. Optional requirement M4(3)(2)(a) applies only where the planning permission under which the building work is carried out specifies that it shall be complied with |

Optional Requirement M4(3) will be met where a new dwelling makes reasonable provision, either at completion or in the future, for a wheelchair user, to live in the dwelling and use any associated private outdoor space, parking and communal facilities that may be provided for the use of the occupants.

Section 3A: Approach to the dwelling

Principal communal entrance	minimum clear width and accessible threshold

and Other communal doors 	300mm “nib” provided to leading edge of door and depth of reveal on leading side of the door not to exceed 200mm

Communal stairs	Ensure that the principal stair that give access to the dwelling meets the provisions of Part K for a general access stair

Section 3B: Private entrances and spaces within, and connected to the dwelling

Principal private entrance 	minimum clear width and accessible threshold

And other external doors 	300mm “nib” provided to leading edge of door and depth of reveal on leading side of the door not to exceed 200mm

Hall and door widths 		minimum clear width and accessible threshold

300mm “nib” provided to leading edge of door and depth of reveal on leading side of the door not to exceed 200mm

Private stairs and changes           all stairs meet the Provisions of Part K

of level within the dwelling          for private stairs.

the stair from the entrance storey to the storey above (or below) and any stair within the storey above (or below), has a clear width of at least 850mm when measured at a height of 450mm above the pitch line of the treads (ignoring any newel post);

Living areas 	glazing to the principal window of this living area starts no higher than 850mm above floor level.